# Transport-Free Module Binding for Sample Preparation using Microﬂuidic Fully Programmable Valve Arrays∗

Gautam Choudhary1,2,†, Sandeep Pal2,†, Debraj Kundu2, Sukanta Bhattacharjee3, Shigeru Yamashita4, Bing Li5, Ulf Schlichtmann5 and Sudip Roy2

1Adobe Research, India, 2CoDA Laboratory, IIT Roorkee, India, 3Dept. of CSE, IIT Guwahati, India,

4CISE, Ritsumeikan University, Japan, and 5Chair of EDA, Technical University of Munich, Germany

Abstract—Microﬂuidic fully programmable valve array (FPVA) biochips have emerged as general-purpose ﬂow-based microﬂuidic lab-on-chips (LoCs). An FPVA supports highly re-configurable on-chip components (modules) in the two- dimensional grid-like structure controlled by some software programs, unlike application-specific ﬂow-based LoCs. Fluids can be loaded into or washed from a cell with the help of ﬂows from the inlet to outlet of an FPVA, whereas cell-to-cell transportation of discrete ﬂuid segment(s) is not precisely possible. The simplest mixing module to realize on an FPVA-based LoC is a four-way mixer consisting of a 2 × 2 array of cells working as a ring-like mixer having four valves. In this paper, we propose a design automation method for sample preparation that finds suitable placements of mixing operations of a mixing tree using four-way mixers without requiring any transportation of ﬂuid(s) between modules. We also propose a heuristic that modifies the mixing tree to reduce the sample preparation time. We have performed an extensive simulation and examined several parameters to determine the performance of the proposed solution.

- Introduction

Microfluidic biochips or Lab-on-Chips (LoCs) are highly in- tegrated analogue of the traditional biochemical laboratory that can perform thousands of experiments using nano-/pico-liter volume of reagents. LoCs redefine the way biological exper- iments are performed by offering a low-cost automated plat- form for point-of-care diagnosis [1], sample preparation [2], genomics [3], and drug discovery [4], [5]. Among several LoCs, flow-based microfluidic biochips (FMBs) manipulate fluids in a network of micro-channels by actuating pressure driven micro-valves [6].

A fully programmable valve array (FPVA) offers a general purpose FMB. An FPVA biochip (Fig. 1) is a two-dimensional array of fluid chambers which is surrounded by up to four microvalves [7]. Valves can be configured to create arbitrary shaped flow paths, mixers, and fluid storage in a programmable fashion (Figs. 1(a)-(b)). The FPVA also supports on-chip components such as fluid metering unit, storage cell, and mixer [7] (Fig. 1(c) and 1(d)).

Any biochemical assay can be modelled as a sequence graph and the process of realizing it on any microfluidic platform is termed as the synthesis of that bioassay. An important step of any bioassay synthesis is module binding, where nodes

†Equal contribution

*This work by D. Kundu and S. Roy was supported partially by the early career research award (grant #: ECR/2016/001921) sponsored by SERB, Govt. of India. This work by B. Li, U. Schlichtmann and S. Roy was supported partially by the Indo-German joint research project (grant #: INT/FRG/DAAD/P-21/2018) sponsored by DST, India and DAAD, Germany.

: Open valve	 : Closed valve	 : FPVA cell

Out-Port

In-Port

Out-Port

In-Port

Out-Port

In-Port

Out-Port

In-Port

(a)	(b)	(c)	(d)

Fig. 1: Paths from in-port to out-port is created by changing valve states before loading of (a) fluid A (pink) and (b) fluid B (orange). (c) Mixing operation of fluids A and B in (3:1) ratio, respectively (one (three) cell(s) is (are) filled with fluid B(A)). (d) storage operation.

of the sequence graph are bound with the instances of on- chip components. Sample preparation is a biochemical assay, where two or more fluids are required to be mixed in a desired ratio [8]. LoC-based sample preparation models the bioassay as a mixing tree to achieve the desired target ratio. Irrespective of any microfluidic platform, storage cells and mixers are required for realizing any mixing tree.

FPVA biochips can perform multiple active mixing in a re- configurable fashion [7]. Several design automation solutions for testing [9] and washing [10] have been proposed by leveraging the above mentioned capabilities of the FPVAs. Although an FPVA supports re-configurable mixer creation and storage of fluids, transportation of precise amounts of fluids from one cell to another cell is challenging (see sec II- B). Note that, with the existing synthesis algorithms for implementing a bioassay on FPVA it is difficult as they require transportation of fluids between cells [11], [12].

In this paper, we propose a practical design automation solution for sample preparation on the FPVA biochips that does not require any transportation of fluids between cells. To the best of our knowledge, the proposed solution considers the constraint of transportation between cells in the FPVA for the first time during sample preparation. We also present a heuristic algorithm to reduce the bioassay completion time. The main contributions of the proposed work are summarized as follows: (1) Transport-free module binding of a mixing tree on an FPVA; and (2) Modifying the initial mixing tree by a heuristic to reduce overall bioassay completion time.

- Background

- Sample Preparation on Microfluidic Biochips

Sample preparation is a process of mixing two or more input reagents in a desired ratio. A sequence of mixing steps,

978-3-9819263-4-7/DATE20/⃝c 2020 EDAA	1335

which is commonly represented using a mixing tree [13], is used to denote sample preparation on microfluidic biochips. An internal (leaf) node in the mixing tree represents a mix operation (input reagent) and an edge weight represents the volume of fluid shared between the nodes. In the literature, several sample preparation algorithms exist for different mi- crofluidic platforms such as digital microfluidic biochips [8],

[14] and FMBs [15]. FMB-based sample preparation platforms are generally equipped with one or more on-chip rotary mixers and storage-cells [13]. A storage-cell is required to store intermediate fluids for subsequent use.

- Fluid Transport Constraint for FPVA

24:18:12:10 M1

M3

2	1  1

M2

3

1	1 2  1

M5	M4

1 2  1	2  2

Reagents : R1, R2, R3, R4 Mi : ith mixing node

(a)

M4

|  | M3 |
| --- | --- |

(c)	(d)

M5

M2

M1

(b)

(e)

In general, an FMB uses carrier fluids such as silicone oil for transporting the payload fluid inside a microchannel. This transportation of a precise volume of fluid requires so- phisticated microstructure called microfluidic latch, a partially closed valve that allows the carrier fluid to pass while blocking the payload fluid [16]. The fluid segmentation within the microchannel of an FMB is a well-known problem while transporting a fluid segment using a carrier fluid [17]. It also poses a significant challenge for an FPVA. In order to move a fluid segment within FPVA from the source cell(s) to the target cell(s) (Fig. 2(a)), a career fluid is required to give the pressure through a flow channel from in-port to out-port connecting the source cell(s) to the target cell(s) (Fig. 2(b)). As the flow channel has several right-angle turns, it results into a non-uniform pressure. Hence, the fluid segment may have breakages as shown in Fig. 2(b). For fluid transportation, a microfluidic latch valve is used at the outlet of the target cell (Fig. 2(b)) that blocks the payload fluid while bypassing the carrier fluid. Thus, in this paper, we consider the transportation of a fluid segment is not possible within an FPVA.

- Motivation of the Work

Out-Port

Source cell

In-Port

(a)

Out-Port

In-Port

(b)

So far, existing sample preparation algorithms for custom FMB mandate transportation of precise volume of fluids. Un- fortunately, the FPVA does not support precise transportation of fluids between cells. Therefore, existing design automation solutions for sample preparation are no longer usable for the FPVAs as they need to transport fluids between cells. However, each cell in the FPVA can store unit volume or takes part in a mixing operation, which can be realized as a rotary mixer by grouping multiple cells together in a re-configurable fashion. In this work, we consider the constraint of transportation between cells in the FPVA, for the first time, and propose

Target cell

: Open valve  : Closed valve

: Microfluidic latch  : FPVA cell

Fig. 2: Fluid transport constraint for an FPVA. (a) Requirement of fluid transportation from source to target cells, and (b) fluid segmentation while transporting a fluid using an immiscible fluid.

Fig. 3: (a) Mixing tree T for 24:18:12:10 obtained by genMixing. (b)-(e) Module binding and scheduling of T in four time cycles.

a design automation approach for sample preparation using FPVA. We leverage the advantage of re-configurable mixing in the FPVA to perform the sample preparation that does not require transportation of fluids between cells.

Example 1: Consider the mixing tree shown in Fig. 3(a) for mixing four input reagents (R1, R2, R3, R4) with a target ratio R1 : R2 : R3 : R4 = 24 : 18 : 12 : 10. The binding of scheduling of mix operations at four different timestamps are shown in Figs. 3(b)-(e). At t = 1, two mixing operations M5 and M2 are bound (Fig. 3(b)). Since M3 mix fluids, which are produced by M5 and M4, we need to bind M4 in a suitable location such that no transportation is required for the mix operation M3. Fig. 3(c) shows the binding of M4 at t = 2. Fig. 3(d) and 3(e) show the module binding of the mix operations M3 and M1 at t = 3 and t = 4, respectively.

- Transport-Free Module Binding for FPVA

For a given mixing tree, the mix operations are scheduled and bound to 2 × 2 modules on the FPVA using an iterative algorithm called as No Transport Mixing (NTM), which does not require any transportation from the storage cells to the corresponding mixers. We propose another method called as Heuristic for Distribution Algorithm (HDA), which transforms the mixing tree for reducing the overall completion time of sample preparation. In this work, we consider genMixing and FloSPA-M to determine the mixing trees as they can exploit all the mixing models for a 2 × 2 mixer [15].

- Problem Formulation

The problem is formulated as follows: Inputs: a mixing tree T , and an FPVA; Output: scheduling and binding of nodes (mix operations) of T with on-chip 2×2 modules; Constraint: cell-to-cell transportation of a fluid after storage is not allowed. Objectives: to minimize (i) the completion time τ of sample preparation, (ii) the total number of cells used C, (iii) the chip-area (estimated as the total number of cells within the minimum bounding rectangle covering C), and (iv) the total number of valves used V , while executing T on the FPVA.

- No Transport Mixing: NTM

The proposed module binding algorithm NTM schedules and binds the mixing nodes of a mixing tree T on the given FPVA. Without loss of generality, the xy-Cartesian coordinate system

# 1336	Design, Automation And Test in Europe (DATE 2020)

C-3,2  C-2,2  C-1,2  C0,2  C1,2  C2,2

C-3,1  C-2,1  C-1,1  C0,1  C1,1  C2,1

M2

C-3,0  C-2,0  C-1,,0  C0,,0  C1,0  C2,0

M1

C-3,-1  C-2,-1  C-1,,-1  C0,,-1  C1,-1  C2,-1

M3

C-3,-2  C-2,-2  C-1,-2  C0,-2  C1,-2  C2,-2

C-3,-3  C-2,-3  C-1,-3  C0,-3  C1,-3  C2,-3

t = 1

M2

R2  R3

M2

R3  R3

M4

M5

R4

R5  R5  R4  R5

M4

R5  R4  R3

M7

(d)

(e)

t = 5

t = 6

M2

M2

R4

4	5

M3

M1

R5

R1  M6

M3

M1 27:25:57:69:78

1	1 1

M2	M3

1	3

R 1	R5

Module binding

4

1

R

R2  R3 1	1	1	1

M4	M5	M6

1	3  1 1	2  2	2

R4	R5	R1 R2  R3

M7	M8

MIX operation

1  2	1  3	1

R3  R4 R5 R1	R2

M

Storage

(a)	(b)	(c)

M

M

t = 2

t = 3

M2

M4

M2

M7

R1  R2

M8

R1  R1

M4

M7  M8

R1  R1

M5

(f)

M

(g)

t = 4

M2

M4  M5

R2  R2

M6

R3  R3

(h)

M

M

M

(i)

(j)

Fig. 4: For a target ratio 27 : 25 : 57 : 69 : 78, (a) mixing tree T obtained by genMixing [15]. (b)-(j) Module binding and time-stamp assignment of the mixing nodes in T on a 6 × 6 FPVA using NTM.

is assumed on the given FPVA cells as shown in Fig. 4(b). For a no-transport mixing we divide the FPVA into quadrants and bind the modules in different quadrants around the origin FPVA cell C0,0. A 2×2 mixers can realize (1:3), (1:1), (1:2:1), and (1:1:1:1) mixing models for on-chip sample preparation [15]. So, we consider only 2 × 2 mixing modules for the module binding. Note that, the four cells corresponding to a 2 × 2 mixing module Mx,y, is determined by the coordinates of the top-right cell of the module, i.e., (x, y).

The first step of NTM is to reorient internal nodes of T , which is denoted as Left Factoring. Given the mixing tree T , Left Factoring reorients the child nodes (mixing node only) of each parent node in decreasing order of their edge weights from left to right in the same level. After the Left Factoring of the mixing tree shown in Fig. 5(a), M3 appears first and then M2. Similarly, M5 appearing first and then M4 (Fig. 5(b)). This transformation facilitate the transport-free module binding without changing the target ratio.

In NTM, we bind the mixing nodes with modules using the following two criteria. 1. Module binding criteria (MBC): A mixing node is allowed to bound only in the quadrants in which its parent node was bound. While binding, the main objective is to bind maximum number of children of a parent node in disjoint quadrants. Children bound to disjoint quad- rants can be executed in parallel. 2. Time-stamp assignment criteria (TAC): When all the inputs of different mixing nodes bound to disjoint quadrants are available, a time-stamp is assigned to the corresponding on-chip modules. All mixing modules with the same time-stamp are executed in parallel.

After left factoring, NTM traverse the mixing nodes of the

tree in BFS order and places them to the FPVA according to MBC. Each time when the TAC is satisfied we assign the time- stamps to the concerned modules. We describe the module binding procedure using the following example.

Example 2: Consider the mixing tree shown in Fig. 4(a). Initially, M1 is bound at M0,0 (Fig. 4(b)), then its child nodes M2 and M3 are bound according to the MBC at M0,1 and M1,−1, respectively (Fig. 4(c)). Next, M4 and M5 (child nodes of M3) are bound at M−1,−1 and M0,−1, respectively (Fig. 4(d)). Although M6 is a child node of M3 it cannot be bound along with M4 and M5. Only M7 (child node of M5) is bound at M1,−1 (Fig. 4(e)). At this stage the TAC is satisfied, so time-stamp t = 1 is assigned to M2, M4 and M7 (Fig. 4(e)). Then M8 (child node of M5) is bound at M2,−1 and since TAC is satisfied time-stamp t = 2 is assigned to M8 (Fig. 4(f)). Now, according to the MBC no other modules can be bound and as time-stamp t = 3 is assigned to M5 (Fig. 4(g)). Eventually, M6 is bound at M1,−2 and time-stamp t = 4 is assigned to M6 (Fig. 4(h)). Next, t = 5 is assigned to M3 (Fig. 4(i)) and finally, t = 6 is assigned to M1 (Fig. 4(j)).

- Heuristic for Distribution Algorithm: HDA

In this section, we propose a heuristic method, called HDA, which transforms the mixing tree and minimizes the sample preparation time as compared to NTM. The idea is to transform the input mixing tree into another one preserving the output ratio such that it balances the number of mix operations bound in each quadrant and parallelize the process. The heuristic is based on the following transformations of a mixing tree [8]:

(a) the nodes (mix operations or input reagents) appearing at the same level of the tree can be permuted, and (b) a node corresponding to an input reagent can be replaced by a mixing node with four copies of the same reagent as its child nodes.

When NTM is applied directly to the mixing tree shown

M2	R1

M1 24:18:12:10

1  1	2

M3

1 2  1

R1 R2 R4  M4

2

1	3

M5

2  1 2

1

R1	R2  R2 R3 R4

(a)

1

M1 24:18:12:10

M3

2

1

M2

1

R1

3

M5

2  1

1

M4  R1 R2 R4

1 2  1

2	2

R2 R3 R4

R1	R2

(b)

in Fig. 6(a), the third quadrant is heavily loaded with mix operations while other quadrants are mostly idle (Gantt chart

Fig. 5: (a) Before and (b) after applying Left Factoring on T .

in Fig. 6(c)). This is due to the fact that the sub-tree rooted at

# Design, Automation And Test in Europe (DATE 2020)	1337

| (a) M1 26:63:47:43:7:70 1 1 1 1 R6 M2 M4 M3 2 2 2 2 R1 R2 R3 R4 1 1 1 1 M5 M6 M7 M8 2 2 1 3 2 1 1 R1 R2 R2 R3 R4 R5 R6 1 1 1 1 M9 M10 M11 M12 2 2 1 3 3 1 2 2 R1 R2 R2 R3 R4 R5 R5 R6 | (b) M1 26:63:47:43:7:70 1 1 1 1 M2 M3 M4 m1 1 1 1 1 1 1 1 1 1 1 1 1 R2 R4 R6 R2 R4 R6 R2 R3 R6 R1 R3 R6 1 1 1 1 M5 M6 M7 M8 1 1 1 2 1 1 1 1 1 1 1 R2 R3 R6 R2 R5 R1 R3 R4 R1 R2 R4 1 1 1 1 M9 M10 M11 M12 1 1 1 1 1 1 1 1 1 1 1 1 1 2 1 R2 R3 R5 R6 R2 R3 R4 R6 R1 R3 R4 R5 R1 R2 R5 | (c) Q4 Q3 Q2 Q1 | M3 | M9 | M10 |  | M11 | M12 | M6 | M7 | M8 | M4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | M5 |  |  |  |  |  |  |  |  |  | M1 |  |
|  |  |  | M2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 2 3 4 5 6 7 8 9 10 Time Cycle |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) M12 M8 m1 Q4 M11 M7 M4 Q3 Q2 M10 M6 M3 M1 Q1 M9 M5 M2 1 T 2 3 4 ime Cycle |  |  |  | (e) footprint chip area |  |  |  |  |  | (f) footprint chip area |  |  |

chip area

Fig. 6: For a target ratio 26 : 63 : 47 : 43 : 7 : 70, mixing tree obtained by (a) genMixing [15] and (b) HDA. Gantt chart, footprint and chip area for (c,e) NTM and (d,f) HDA+NTM for the mixing tree shown in Fig. 6(a).

M4 contains almost all mixing nodes. As a result the sample preparation completes in 10 time cycles. HDA transforms the mixing tree shown in Fig. 6(a) into an equivalent tree shown in Fig. 6(b), leveraging ratio preserving transformations. The Gantt chart for HDA + NTM, i.e., applying NTM to the transformed mixing tree by HDA (Fig. 6(d)) shows that the loads of each quadrant are balanced and the sample preparation time decreases to 4 time cycles. The footprint (of the cells used) and the chip area (estimated by the minimum bounding rectangle) of an FPVA for NTM and HDA + NTM are shown in Figs. 6(e) and 6(f), respectively.

- Simulation Results

We perform an extensive simulation to analyze the per- formance of the proposed algorithm. In our experiments, we consider a set of 50,000 or maximum possible (whichever is lower) ratios for all ratio sum L = 64, 256 by varying the number of reactant fluids K = 6, 7, 8, 9, and compute six performance parameters O (number of mixing operations), τ (time cycles for completion), C (number of cells used), V (number of valves used), A (total actuations of valves for mixing), and B (area of the minimum bounding rectangle). The proposed NTM is run without and with HDA over the large set of target ratios to evaluate the parameter values. Table I shows the values of all these parameters for L = 64 and L = 256. Note that, HDA+NTM increases the average number of mixing operations (Oavg) as it introduces new mix nodes. Therefore, HDA+NTM requires more valve actuations (Aavg) for performing the extra mixing operations. However, the average numbers of cycles (τavg), cells used (Cavg), and valves used (Vavg) are reduced by HDA+NTM. Reductions

- Conclusions

We present a technique for design automation for sample preparation on FPVA chip. The proposed method can realize any mixing tree on the FPVA without using any fluid trans- portation between any two cells. We also propose a heuristic that distributes mixing operations in a mixing tree to reduce the overall assay operation time and chip area for the FPVA.

References

- V. Gubala et al., “Point of Care Diagnostics: Status and Future,” Anal. Chem., vol. 84, no. 2, pp. 487–515, 2012.

- R. H. Liu et al., “Self-Contained, Fully Integrated Biochip for Sample Preparation, Polymerase Chain Reaction Amplification, and DNA Mi- croarray Detection,” Anal. Chem., vol. 76, no. 7, pp. 1824–1831, 2004.

- H. Yin et al., “Microfluidics for Single Cell Analysis,” Current Opinion in Biotechnology, vol. 23, no. 1, pp. 110–119, 2012.

- C. D. Chin et al., “Microfluidics-Based Diagnostics of Infectious Diseases in the Developing World,” Nat Med., vol. 17, no. 8, pp. 1015– 1019, 2011.

- S. Einav et al., “Discovery of a Hepatitis C Target and its Pharmaco- logical Inhibitors by Microfluidic Affinity Analysis,” Nat Biotechnol., vol. 26, no. 9, pp. 1019–1027, 2008.

- J. Melin et al., “Microfluidic Large-Scale Integration: The Evolution of Design Rules for Biological Automation,” Annual Review of Biophysics and Biomolecular Structure, vol. 36, no. 1, pp. 213–231, 2007.

- L. M. Fidalgo et al., “A Software-Programmable Microfluidic Device for Automated Biology,” Lab Chip, vol. 11, pp. 1612–1619, 2011.

- S. Bhattacharjee et al., Algorithms for Sample Preparation with Mi- crofluidic Lab-on-Chip. River Publishers, 2019.

- C. Liu et al., “Testing Microfluidic Fully Programmable Valve Arrays (FPVAs),” in Proc. DATE, 2017, pp. 91–96.

- Y. Lin et al., “Block-Flushing: A Block-Based Washing Algorithm for Programmable Microfluidic Devices,” in Proc. DATE, 2019, pp. 1531– 1536.

- T. Tseng et al., “Reliability-Aware Synthesis With Dynamic Device Mapping and Fluid Routing for Flow-Based Microfluidic Biochips,” IEEE TCAD, vol. 35, no. 12, pp. 1981–1994, 2016.

- A. Grimmer et al., “Close-to-optimal placement and routing for continuous-flow microfluidic biochips,” in Proc. ASP-DAC, 2017, pp. 530–535.

- W. Thies et al., “Abstraction Layers for Scalable Microfluidic Biocom- puting,” Natural Computing, vol. 7, no. 2, pp. 255–275, 2008.

- S. Roy et al., “Optimization of Dilution and Mixing of Biochemical Samples using Digital Microfluidic Biochips,” IEEE TCAD, vol. 29,

in the τavg

, i.e., sample preparation time, and C

avg

, Vavg

are

no. 11, pp. 1696–1708, 2010.

- S. Bhattacharjee et al., “Dilution and Mixing Algorithms for Flow-

because HDA+NTM can exploit parallelism by distributing mixing nodes into different quadrants of the FPVA. Moreover, HDA+NTM requires less chip area (Bavg) (estimated as the minimum area rectangular bounding box) compared to NTM.

Based Microfluidic Biochips,” IEEE TCAD, vol. 36, no. 4, pp. 614–627, 2017.

- J. P. Urbanski et al., “Digital microfluidics using soft lithography,” Lab Chip, vol. 6, no. 1, pp. 96–104, 2006.

- T. Fu et al., “Bubble Formation and Breakup Dynamics in Microfluidic Devices: A Review,” CES, vol. 135, pp. 343–372, 2015.

TABLE I: NTM vs. (HDA + NTM) on the basis of Oavg , τavg , Cavg , Vavg , Aavg and Bavg for L = 64 and 256 over # reagents K = 6, 7, 8, and 9.

|  | L = |  |  |  |  |  | 64 |  |  |  |  |  | L = |  |  |  |  |  | 256 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K | NTM |  |  |  |  |  | HDA+NTM |  |  |  |  |  | NTM |  |  |  |  |  | HDA+NTM |  |  |  |  |  |
|  | Oavg | τavg | Cavg | Vavg | Aavg | Bavg | Oavg | τavg | Cavg | Vavg | Aavg | Bavg | Oavg | τavg | Cavg | Vavg | Aavg | Bavg | Oavg | τavg | Cavg | Vavg | Aavg | Bavg |
| 6 | 5.6 | 3.7 | 13.0 | 17.1 | 22.5 | 17.9 | 6.3 | 3.0 | 10.6 | 13.2 | 25.1 | 12.7 | 6.4 | 4.6 | 12.4 | 16.6 | 25.7 | 17.1 | 7.6 | 4.0 | 8.8 | 10.9 | 30.4 | 9.7 |
| 7 | 6.3 | 4.2 | 14.5 | 19.2 | 25.1 | 20.3 | 7.0 | 3.0 | 12.0 | 15.0 | 28.0 | 14.4 | 6.7 | 4.9 | 13.3 | 17.8 | 27.0 | 18.9 | 8.1 | 4.0 | 9.5 | 11.9 | 32.5 | 11.1 |
| 8 | 6.8 | 4.6 | 15.6 | 20.7 | 27.3 | 22.0 | 7.6 | 3.0 | 13.2 | 16.5 | 30.4 | 15.4 | 7.0 | 5.1 | 14.0 | 18.9 | 28.3 | 20.4 | 8.7 | 4.0 | 10.3 | 12.9 | 34.9 | 12.5 |
| 9 | 7.1 | 4.8 | 16.1 | 21.5 | 28.6 | 22.6 | 7.9 | 3.0 | 13.9 | 17.4 | 31.8 | 15.9 | 7.4 | 5.4 | 14.7 | 20.0 | 29.7 | 21.8 | 9.4 | 4.0 | 11.2 | 14.0 | 37.6 | 13.9 |

1338	Design, Automation And Test in Europe (DATE 2020)